ECA0806 - ANALOG AND DIGITAL COMMUNICATION

NAME: JAYASREE S

REG NO: 192512299

EXPERIMENT 3

PROGRAM:

clc;

clear;

close all;

% Parameters

fs = 100000; % Sampling Frequency

t = 0:1/fs:0.02; % Time Vector

fm = 500; % Message Frequency

fc = 5000; % Carrier Frequency

% Message Signal

m = cos(2*pi*fm*t);

% Carrier Signal

c = cos(2*pi*fc*t);

% DSB-SC Modulation

dsb = m .* c;

% VSB Filter (Low-pass + High-pass Approximation)

[b,a] = butter(4, fc/(fs/2));

vsb = filter(b,a,dsb);

% Coherent Demodulation

demod = vsb .* (2*cos(2*pi*fc*t));

% Recover Message

[b1,a1] = butter(4, 1000/(fs/2));

rec = filter(b1,a1,demod);

% Display Results

figure;

subplot(5,1,1)

plot(t,m)

title('Message Signal')

grid on

subplot(5,1,2)

plot(t,c)

title('Carrier Signal')

grid on

subplot(5,1,3)

plot(t,dsb)

title('DSB-SC Signal')

grid on

subplot(5,1,4)

plot(t,vsb)

title('VSB Signal')

grid on

subplot(5,1,5)

plot(t,rec,'r')

title('Recovered Message')

grid on